Dataset and Data Generation

We construct a synthetic dataset to study the inverse problem of reconstructing dense 3D structural deformation from sparse 2D observations. Each sample in the dataset corresponds to an independent vibration scenario of a cantilever beam with randomized observation and imaging configurations.

Geometry and Displacement Field

The underlying structure is represented by a triangular mesh with N=1538 vertices. Let X0∈RN×3 denote the rest configuration. The dynamic displacement field over time is defined as: Ut=ϕ⋅at, where ϕ∈RN×3 is a spatial mode shape and at is a temporal amplitude following a sinusoidal pattern: at=Asin⁡(ωt+φ). The maximum displacement magnitude is approximately 0.02∼0.03 meters. Each sequence contains T=200 frames.

Sparse 2D Observations

At each time step t, only a subset of K=20 vertices are observed. The 3D displacements of these vertices are projected into the image plane using a pinhole camera model: yt∈RK×2. The observed signal is expressed as normalized pixel displacement: y=Δpixelfx, where fx is the focal length. The resulting signal magnitude is on the order of 10-3, corresponding to approximately 1–7 pixels in image space. Small Gaussian noise is added to simulate measurement uncertainty.

Randomization and Sample Diversity

To ensure generalization, each sample is generated with independent randomization in the following aspects:

- Observation points: The set of K=20 observed vertices is randomly selected from the full mesh.

- Camera configuration: Camera position and orientation are randomly sampled, resulting in varying projection geometries.

- Vibration parameters: Amplitude A, frequency ω, and phase φare randomly sampled for each sequence.

- Measurement noise: Low-level Gaussian noise is added to the 2D observations.

Training and Testing Split

The dataset is organized at the sample level, where each sample corresponds to a full vibration sequence:

- Training set: 100 samples

- Testing set: 30 samples

Each sample contains 200 frames, resulting in: Training frames=100×200=20,000.

During training, each frame (yt,Ut) is treated as an independent data point. Importantly, there is no overlap between training and testing configurations:

- different observed vertex sets

- different camera parameters

- different vibration parameters

This ensures a fair evaluation of generalization to unseen conditions.

Problem Formulation

The learning task is defined as: f:yt∈RK×2→Ut∈RN×3, which is an ill-posed inverse problem due to:

- sparse observations (K≪N)

- dimensionality mismatch (2D to 3D)

- ambiguity in projection

Training

We learn a mapping from sparse 2D observations to dense 3D displacement fields using a probabilistic neural network: fθ:yt→Ut where: yt∈RK×2is the sparse observation. Ut∈RN×3is the dense displacement field. Each frame is treated as an independent training sample. The training data is constructed at the frame level.

Each vibration sequence is decomposed into individual frames, and each frame contributes one training sample. For each sample:

- input is flattened:yt∈RK×2→R2K

- Target is flattened:Ut∈RN×3→R3N

All samples from all training sequences are concatenated into a single dataset.

To address scale mismatch between input and output, both observations and displacement fields are normalized:

y=y-μyσy,U=U-μUσU

The normalization statistics are computed over the entire training dataset and reused during inference.

The network predicts both the mean and uncertainty of the displacement field: Uμ,logσ2, where: Uμ is the predicted mean displacement and σ2 represents per-vertex uncertainty. A latent variable z is introduced to model ambiguity in the inverse mapping.

We train the model using a probabilistic objective consisting of a Gaussian negative log-likelihood and a KL divergence term: L=λnll⋅LNLL+βKL⋅LKL

Model Architecture

We model the mapping from sparse 2D observations to dense 3D displacement fields using a conditional probabilistic neural network. Given input yt∈R2K, the model predicts a distribution over the displacement field: p(Ut∣yt).

Latent Representation

To capture ambiguity in the inverse problem, we introduce a latent variable z: z∼q(z∣yt), where q(z∣yt) is parameterized as a Gaussian distribution: q(z∣yt)=N(μz,σz2). The encoder network maps the input observation to the latent parameters:

μz,logσz2)=Encoder(yt

Decoder

The decoder predicts the displacement distribution conditioned on both yt and z:

Uμ,logσU2)=Decoder(yt,z

where:

- Uμ∈R3N: predicted mean displacement

- σU2∈R3N: predicted uncertainty

Reparameterization

During training, the latent variable is sampled using the reparameterization trick:

z=μz+σz⋅ϵ,  ϵ∼N(0,I)

Network Structure

Both encoder and decoder are implemented as MLPs with fully connected layers and nonlinear activations.

Evaluation Protocol

Inference Procedure

At test time, the model takes sparse 2D observations from a single frame: yt∈RK×2 and predicts a distribution over the dense 3D displacement field: p(Ut∣yt). The model outputs:

- mean displacement Uμ

- per-vertex uncertainty σU

Sequence-level Reconstruction

Although the model operates on single frames, full sequences are reconstructed by applying the model independently to each frame: ∀t,Ut=fθ(yt). This produces:

- predicted displacement sequence Upred∈RT×N×3

- predicted uncertainty Ustd∈RT×N×3

Monte Carlo Inference

To account for latent uncertainty, we perform Monte Carlo sampling during inference. For each frame, multiple forward passes are performed: Uss=1S.

The final prediction is computed as: E[U]=1SsUs and total uncertainty is estimated as: Var(U)=E[σ2]+Var(Us).

This captures both:

- aleatoric uncertainty (from predicted variance)

- epistemic uncertainty (from latent sampling)

Evaluation Strategy

We evaluate the model on a held-out test set consisting of 30 samples. Each sample represents a distinct vibration scenario with different:

- observed vertex sets

- camera configurations

- vibration parameters

There is no overlap between training and testing conditions. Metrics are computed for each sample independently and then averaged across all test samples.

Metrics

1. Global Error: RMSE, MAE, Relative L2.

2. Axis-wise Error: RMSEx,y,z,MAEx,y,z.

3. Vertex-level Error: ∥Upred-Ugt∥2.

4. Temporal Consistency: We evaluate the correlation between predicted and ground-truth vibration curves at the vertex with maximum displacement: Corr(Upred,Ugt).

Uncertainty Evaluation

1. Coverage: We measure how often the ground truth lies within predicted uncertainty bounds: ∣error∣≤k⋅σ, for k=1,2,3

2. Error–Uncertainty Correlation: We compute correlation between prediction error and predicted uncertainty: Corr(error,σ)

3. Confidence Filtering: We evaluate performance under confidence-based filtering. Vertices are sorted by predicted uncertainty, and only the top p%most confident predictions are retained. Metrics are computed for: p∈{10%,20%,50%,80%,100%}

4. Negative Log-Likelihood: We also report the Gaussian negative log-likelihood: LNLL to evaluate the quality of predicted uncertainty distributions.